# REQUEST FOR QUOTATION

Ref.:   RFQ/KOS/ 620179/2/2021

Subject: Supply and delivery of different items for webinar room at Kosovo Academy for Public Safety (KAPS)”

The OSCE has a requirement for the supply with items/goods listed below and kindly requests you to provide a quotation as follows:

Please use this form for the submission of your quotation.  Additional information can be supplied on a separate page.  Technical specification sheets, brochures, photos, if applicable need to be submitted together with the quotation.

| Item No. | Description | Qty | Currency | Unit Price (excl. VAT) | Total (excl. VAT) |
| --- | --- | --- | --- | --- | --- |
| 1. | Supply and delivery of interactive display: Diagonal Size: 75" Aspect Ratio: 16:9 Screen Type: LCD Direct LED Backlight Resolution: 4K UHD (3840x2160 @60Hz) Response Time: 8ms Refresh Rate: minimum 60Hz Angle View: 178˚ Contrast Ratio: 1200:1 Dynamic Contrast Ratio: 5000:1 Must have Tempered Glass MOHS L7 Must have Heat Tempered and Antiglare Glass Type Brightness: minimum 400cd/m2 Must support integration of Intel Computer using OPS SlotMust have minimum 3 HDMI Input ports, 1 HDMI Output port, 2 USB-A 2.0 ports, 3 USB A 3.0 ports, 2 USB C ports, 1000Mbps port for LAN Access, Wireless Adapter, Bluetooth integration, 1 Display port as input, 1 VGA Input port including VGA Audio, 1 Optical Out port for Audio and 1 Headphone port Must have High precision Writing technology Must support minimum 20 continuous touch points Touch resolution must be at least 32768x32768 pixels with a response time less than 8ms Touch accuracy must be 1.5mm Simultaneous Writing must be supported Must be offered with Pen Holder and dual Recognitions Stylus pen Must support Pen & Touch differentiation Must support tracking rate of minimum 6m/s The Interactive TV must be offered with Android Must have at least 6GB RAM Memory Must support at least 64GB Internal Memory Must be offered with Quad Core CPU with at least A73 CPU Must be offered with Quad Core MALI G52 GPU Android resolution must be 4K minimum Must have minimum 2 x 15W Speakers including Subwoofer for better audio experience Equipment must be offered with Wall mounting Kit. Equipment must be offered with Installation services and setup. System must be offered including Timer APP, Media Player APP, Web Browser APP, PDF Reader APP, Screen Recording APP, Voting APP, Math’s Tools APP .Basic training must be included in the offer. | 1pcs | € |  |  |
| 2. | Supply and delivery of PC computer: Desktop PC Minimal Technical Specification, Processor: Min. Core i5, 10th Generation Memory: Min. 8GB DDR4 2666Mhz Storage: Min. 512GB SSD M.2 NVMe,Graphics: Intel UHD Graphics,I/O Ports: Min. 1x RJ-45, 2x USB 3.1, 2x USB 2.0, 10/100/1000 Gigabit LAN, Bluetooth 4.0, WiFi 5, Brand Name Keyboard & Mouse | 2pcs | € |  |  |
| 3. | Supply and delivery of Full HD Monitors 1920x1080 2ms HDMI DVI Back-lit LED Monitor Min. 23" LED Resolution: Min. 1920x1080Connectivity: Min. 1x HDMI, 1X VGA | 4pcs | € |  |  |
| 4. | Supply and delivery Wireless Headsets: Wireless headphone closed or in-ear or statoscope type, with radio frequency receiver. UHF Wireless Microphone System Headset Mic/Stand Mic/Lavalier Lapel Mic with Rechargeable Bodypack Transmitter & Receiver 1/4" Output Up to 900MHz frequency（902MHz-914.4MHz） The Wireless Transmission Distances Up to 100 Feet - you can move freely in the classroom, conference room, stage, and outdoors. With LCD display - The LCD display on the Bodypack transmitter can display and adjust the frequency, microphone volume, and battery level. Both the body pack transmitter and the portable mini receiver are powered by a rechargeable battery. | 19pcs | € |  |  |
| 5. | Supply and delivery Wireless microphone: UHF Rechargeable Wireless Microphone with Bluetooth Receiver System Wireless Band Used UHF Band Microphone Type Handheld Dynamic Polar Pattern Unidirectional Wireless Operating Range 165' / 50.3 m Frequency Stability < 0.002% Sensitivity 2 mW Signal to Noise Ratio > 95 dB Transmitted Power 10 mW Dynamic Range > 95 dB Distortion < 1%, 1 kHz Modulation Frequency Response ± 3dB 50 Hz to 18 kHz (±3 dB) Construction Material Metal Alloy Receiver Base, ABS and Metal Microphone | 1pcs | € |  |  |
| 6. | Supply and delivery of Condenser Microphone & AI-1 One-Channel USB Audio Interface Pack Mounting arm | 1pcs | € |  |  |
| 7. | Supply and delivery of Webcams 1080p USB suitable for live streaming. Up to Full HD 1080p Resolution Min. 30 fps Frame rate Field of View 78° Diagonal Auto Focus, Glass Lens Mics Dual Omnidirectional NC (Noise Cancellation) Technology Auto-Light Correction Tripod and Clip Thread Mounting Options Plug-and-Play USB Cable Windows, Mac, and Chrome Compatible | 2pcs | € |  |  |
| 8. | Supply and delivery of acoustic panels for isolation of sound at the webinar room. Thickness : 5 cm Width : 90 cm Length : 180 cm Weight : 16kg/m3 | 4pcs | € |  |  |
| 9. | Supply and delivery of ON AIR - LED Illuminated Sign for webinar room On air sign with back light and switch. uses LEDs as the light source and displays "ON AIR" horizontally in bright red when turned on. When off, it's totally blacked out, leaving no confusion about whether it's off. It has a solid steel frame and a thick, red plexiglass faceplate. Use it mounted on standard, single gang boxes or with wall anchors. Size (LxW×H): Approx. 30×15×10cm | 1pcs | € |  |  |
| 10. | Supply and delivery of Dual Monitor Mount for 13-27 inch LCD LED Monitor Computer PC Screens, Ergonomic Double Arms Stand Desk Mount Bracket Tilt 90° Swivel 180° Rotate 360°, | 2pcs | € |  |  |
| 11. | Delivery cost DAP OSCE MIK Warehouse, Fushe Kosove/Kosovo Polje 12000, Kosovo | Lump sum – in Euro without VAT (please specify) |  |  |  |
| TOTAL : Amount (in €) without VAT |  |  |  |  |  |

Photo for illustration purpose of the webinar room

NOTE: OSCE will award the lowest financial (technically compliant) Bid.

Any Purchase Order will be subject to the OSCE General Conditions of Contract which can be viewed at http://www.osce.org/procurement.  Detailed “Instructions to Bidders – Request for Quotation” are also provided on this website.

Deadline for submission of quotations: on or before 15 October, 2021 at 12:00hrs (CET)

QUOTATIONS SHALL BE ADDRESSED AND SUBMITTED TO:  Tenders.OMiK@osce.org

The technical compliance form (below) as well as brochures, photos, of the offered goods need to be separated from financial offer (above).

Delivery time: (please specify)             	……………………………………………….

Delivery terms:                             DAP OSCE MIK Warehouse, Fushe Kosove/Kosovo Polje

12000, Kosovo

Payment terms:                            30 days net

Validity of offer (60 days minimum):      .……………………………………………….

Company name:

Authorized representative’s name and signature:

Address:

Email:

Telephone

ANNEX A: TECHNICAL COMPLIANCE FORM

Ref Number: RFQ/KOS/ 620179/2/2021.

Kindly submit the technical compliance form (below) separately from the financial offer. This requirement is mandatory

Nr.; Technical Specifications; Compliance Remarks The Bidders are required to put item-by-item commentary on the Technical Specifications/Requirements demonstrating substantial responsiveness of the Goods to those specifications/requirements, or a statement of deviations and exceptions to the provisions of the Technical Specifications/Requirements. DO NOT INCLUDE ANY FINANCIAL INFORMATION IN THIS FORM. 1.; Supply and delivery of interactive display: Diagonal Size: 75" Aspect Ratio: 16:9 Screen Type: LCD Direct LED Backlight Resolution: 4K UHD (3840x2160 @60Hz) Response Time: 8ms Refresh Rate: minimum 60Hz Angle View: 178˚ Contrast Ratio: 1200:1 Dynamic Contrast Ratio: 5000:1 Must have Tempered Glass MOHS L7 Must have Heat Tempered and Antiglare Glass Type Brightness: minimum 400cd/m2 Must support integration of Intel Computer using OPS Slot Must have minimum 3 HDMI Input ports, 1 HDMI Output port, 2 USB-A 2.0 ports, 3 USB A 3.0 ports, 2 USB C ports, 1000Mbps port for LAN Access, Wireless Adapter, Bluetooth integration, 1 Display port as input, 1 VGA Input port including VGA Audio, 1 Optical Out port for Audio and 1 Headphone port Must have High precision Writing technology Must support minimum 20 continuous touch points Touch resolution must be at least 32768x32768 pixels with a response time less than 8ms Touch accuracy must be 1.5mm Simultaneous Writing must be supported Must be offered with Pen Holder and dual Recognitions Stylus pen Must support Pen & Touch differentiation Must support tracking rate of minimum 6m/s The Interactive TV must be offered with Android Must have at least 6GB RAM Memory Must support at least 64GB Internal Memory Must be offered with Quad Core CPU with at least A73 CPU Must be offered with Quad Core MALI G52 GPU Android resolution must be 4K minimum Must have minimum 2 x 15W Speakers including Subwoofer for better audio experience Equipment must be offered with Wall mounting Kit. Equipment must be offered with Installation services and setup. System must be offered including Timer APP, Media Player APP, Web Browser APP, PDF Reader APP, Screen Recording APP, Voting APP, Math’s Tools APP .Basic training must be included in the offer.. 2.; Supply and delivery of PC computer: Desktop PC Minimal Technical Specification, Processor: Min. Core i5, 10th Generation Memory: Min. 8GB DDR4 2666Mhz Storage: Min. 512GB SSD M.2 NVMe, Graphics: Intel UHD Graphics, I/O Ports: Min. 1x RJ-45, 2x USB 3.1, 2x USB 2.0, 10/100/1000 Gigabit LAN, Bluetooth 4.0, WiFi 5, Brand Name Keyboard & Mouse. 3.; Supply and delivery of Full HD Monitors 1920x1080 2ms HDMI DVI Back-lit LED Monitor Min. 23" LED Resolution: Min. 1920x1080Connectivity: Min. 1x HDMI, 1X VGA. 4.; Supply and delivery Wireless Headsets: Wireless headphone closed or in-ear or statoscope type, with radio frequency receiver. UHF Wireless Microphone System Headset Mic/Stand Mic/Lavalier Lapel Mic with Rechargeable Bodypack Transmitter & Receiver 1/4" Output Up to 900MHz frequency（902MHz-914.4MHz） The Wireless Transmission Distances Up to 100 Feet - you can move freely in the classroom, conference room, stage, and outdoors. With LCD display - The LCD display on the Bodypack transmitter can display and adjust the frequency, microphone volume, and battery level. Both the body pack transmitter and the portable mini receiver are powered by a rechargeable battery.. 5.; Supply and delivery Wireless microphone: UHF Rechargeable Wireless Microphone with Bluetooth Receiver System Wireless Band Used UHF Band Microphone Type Handheld Dynamic Polar Pattern Unidirectional Wireless Operating Range 165' / 50.3 m Frequency Stability < 0.002% Sensitivity 2 mW Signal to Noise Ratio > 95 dB Transmitted Power 10 mW Dynamic Range > 95 dB Distortion < 1%, 1 kHz Modulation Frequency Response ± 3dB 50 Hz to 18 kHz (±3 dB) Construction Material Metal Alloy Receiver Base, ABS and Metal Microphone. 6.; Supply and delivery of Condenser Microphone & AI-1 One-Channel USB Audio Interface Pack Mounting arm. 7.; Supply and delivery of 1080p USB Webcams suitable for live streaming. Up to Full HD 1080p Resolution Min. 30 fps Frame rate Field of View 78° Diagonal Auto Focus, Glass Lens Mics Dual Omnidirectional NC (Noise Cancellation) Technology Auto-Light Correction Tripod and Clip Thread Mounting Options Plug-and-Play USB Cable Windows, Mac, and Chrome Compatible. 8.; Supply, and delivery of acoustic panels for isolation of sound at the webinar room. Thickness : 5 cm Width : 90 cm Length : 180 cm Weight : 16kg/m3. 9.; ON AIR - LED Illuminated Sign for webinar room On air sign with back light and switch. uses LEDs as the light source and displays "ON AIR" horizontally in bright red when turned on. When off, it's totally blacked out, leaving no confusion about whether it's off. It has a solid steel frame and a thick, red plexiglass faceplate. Use it mounted on standard, single gang boxes or with wall anchors. Size (LxW×H): Approx.30×15×10cm. 10.; Supply and delivery of Dual Monitor Mount for 13-27 inch LCD LED Monitor Computer PC Screens, Ergonomic Double Arms Stand Desk Mount Bracket Tilt 90° Swivel 180° Rotate 360°,. 11.; Delivery Schedule: Delivery at DAP OSCE MIK Warehouse, Fushe Kosove/Kosovo Polje 12000, Kosovo (No partial delivery is permitted). Please confirm compliance and specify the offered delivery period.. Warranty: Warranty period should be minimum 2 years for all products under this RFQ. Please confirm compliance and specify the offered warranty terms.

Authorized representative’s name and signature (stamp) ___________________________