Annex II. Technical Specifications and Schedule of Requirements

| # | Technical Specifications/Description | Compliance Remarks The Bidders are required to specify make/model and put item-by-item commentary on the Technical Specifications demonstrating substantial responsiveness of the Goods to those specifications, or a statement of deviations and exceptions to the provisions of the Technical Specifications. |
| --- | --- | --- |
| GENERAL SPECIFICATIONS: |  |  |
| 1. | Disinfection unit on trailer chassis: The disinfection chamber (steam-formalin chamber) is intended for disinfection of cloth, wool and leather, bedding and other things using steam-air and steam-formalin methods. Disinfection of the clothes and beddings is carried out automatically. The chamber compartment with a volume of 2.4 m³, bedding is disinfected with steam method or formalin-vapour disinfection. The disinfection chamber should be supplied with a set of technical and operational documentation in Russian. Technical Specifications: Length, mm: 4000-4500 Width, mm: 2000-2500 Height, mm: 2300-2800 Weight of the equipped unit, kg: 1400-1800 Quantity: 1 (one) unit. Chassis: Spring Brake: Overrun Lighting System A. Exterior Lights Type: LED Voltage: 12V DC Color: Red (rear), Amber (front and sides) License Plate Light: Chassis Frame Material: Type: Steel or aluminum Coating: Corrosion-resistant coating Axles A. Type Single or Tandem: Single Axle: For lighter trailers Tandem Axle: For heavier trailers Wheels and Tires A. Wheels Material: Steel or aluminum alloy Size: Common sizes include 14", 15", or 16" diameter Additional Features A. Jack Type: Manual or electric tongue jack Capacity: Suitable for the trailer's tongue weight Disinfection Chamber part details: -Design: rectangular type, double sided loading and unloading from opposite sides; an umbrella inside on top, there are two rails with hangers for the items being processed; the evaporator is located downside. -Camera material: Chamber shell and doors are made of steel. -Disinfection chamber volume not less than, m3: 2.5 -Floor area of the disinfection chamber is not less than, m2: 1.7 Please see below the sample photo of disinfection unit on trailer chassis: |  |
| 2. | Copy of manufacture certificate, if available |  |

| ## | Other Mandatory Requirements | Compliance Remarks The Bidders are required provide item-by-item commentary on the Technical Specifications demonstrating substantial responsiveness of the goods to those specifications (e.g. put “comply”), or a statement of deviations and exceptions to the provisions of the Technical Specifications |
| --- | --- | --- |
| 1. | All requested goods should be in new condition, never used before, packed in manufacture’s packages (Please confirm) |  |
| 2. | Terms of delivery – DAP/DDP, OSCE Programme Office in Dushanbe, located at Ahmadi Donish 18A, Dushanbe, Tajikistan (please confirm). |  |
| 3. | Delivery Schedule: 30 days after singing of contract. (Please confirm or specify in case of deviation) |  |
| 4. | Acceptance of OSCE General Conditions of Contract (please confirm) |  |
| 5. | Payment terms: 30 days net after provision of goods/services (Please confirm or specify in case of deviation) |  |
| 6. | Mode of payment (bank transfer): [please confirm] |  |
| 7. | Validity of offer 60 days minimum: (Please confirm or specify in case of deviation) |  |
| 8. | Warranty period for goods: minimum 12 months. (Please confirm or specify in case of deviation) |  |
| 9. | Please provide photos for proposed equipment, if possible. |  |

Company name:

Authorized representative’s name and stamp/signature:

Address:

Email:

Telephone: